隧道掘进机（Tunnel Boring Machine，简称TBM）因其高效、安全、被广泛应用于硬岩隧道挖掘工作中1-3。作为TBM破岩的关键部件，由于硬岩的高强度、高磨损特性，盘型滚刀在破碎岩石的过程中逐渐磨损，当磨损量累积到一定程度时，TBM的破岩效率下降，如不及时更换滚刀，会对滚刀产生过大的轴向载荷，可能进一步导致轴承、密封件甚至整个刀盘的破坏，更会加速周围滚刀的磨损，这不仅降低了TBM的掘进效率，还会带来重大的施工风险。如能及时获取滚刀磨损信息，从而改进当前TBM掘进策略，以降低滚刀磨损速度，进一步的减少换刀频率，能有效提高掘进效率，因此，准确获取滚刀的磨损信息具有重要意义4。

常用的滚刀磨损检测方法有三种：刀盘内部检测法（inside-cutterhead inspection)，在线监测法（online monitoring）以及滚刀磨损预测法（wear prediction）5。其中刀盘内部检测法是检测滚刀磨损情况最直接、最有效的方法，也是实际工程中最普遍采用的方法。在TBM几个工作周期后，施工人员进入刀盘区域，直接使用游标卡尺、深度尺等工具测量滚刀磨损尺寸，并观察刀盘表面裂纹情况以确认刀盘磨损情况6。然而这种方法需要不断地停机检查，严重影响施工进度，一般来说，换刀、检查、维修的时间约占整个工期的1/37。在线监测法通过传感器实时监测滚刀磨损状态8，能及时发现滚刀磨损，常用方法有超声波监测9，涡流检测10,11等，但是在TBM高温（70-90℃），高湿度（80%-90%相对湿度），强烈振动（10-20倍重力加速度）的恶劣环境下12，该方法易产生较大误差。同时该方法成本昂贵，据调查，在不考虑工作环境对传感器造成损坏的情况下，使用该方法的总成本在10万美元左右6。

滚刀磨损预测法是直接预测滚刀消耗量，一般来说，滚刀磨损预测方法主要分为两种：经验方法和半理论方法。经验方法在深度学习方法未成熟之前，学者们主要通过先验选择地质参数，使用线性回归的方法研究其与滚刀磨损量之间的函数关系。其中最经典的模型是由科罗拉多矿业学院开发的CSM模型和挪威理工学院开发的NTTU模型，二者在很多工程中得到验证，因此被广泛应用于实际工程之中，其中CSM模型使用CAI值、计算的TBM掘进速率和TBM工作时间或挖掘岩石体积来预测滚刀更换时间，NTTU模型使用专门的磨蚀性值（AV）来估计滚刀寿命。除此之外，Hassanpour等使用戈姆鲁德输水隧道（GWCT）和伊朗Karaj输水隧道的挖掘数据，先后开发了火山碎屑岩和镁铁质火成岩4、沉积岩和中低品位变质岩13等地质条件适用的滚刀磨损预测模型。Ghodrat Barzegari 等针对Naghadeh输水隧道项目，开发了使用如 Cerchar 磨损指数(CAI)和单轴抗压强度 （UCS）等常见岩石参数预测磨损率的模型14，该模型在花岗岩地质条件下的磨损预测性能优于传统CSM，NTTU等模型。但由于上述方法通常使用简单的线性回归或多元线性回归及其变体建立预测模型，建立的模型精度通常不高。并且由于需要从岩石的单轴抗压强度（UCS）、Cerchar磨蚀性指数（CAI）等地质参数中获取滚刀磨损信息，因此强烈的依赖于准确的地质参数，但实际工程中地质参数难以获取。同时上述模型都是基于特定案例开发的模型，只适用于相同或相似的地质条件，进一步的限制其应用。

而半理论方法大多基于滚刀破岩机制，磨损机制，以及能量分析，以探究滚刀磨损机制和滚刀布置规律。例如，Wang等通过分析滚刀和硬岩之间的摩擦过程，推导出TBM刀盘系统的能量方程，并结合磨损能量理论，提出了基于能量分析的硬岩 TBM 刀盘的滚刀磨损预测方法，最后利用了引洮工程9号隧道的工程数据验证了模型的有效性15。Wang通过研究滚刀在切割硬岩时的机械过程，建立了新的比能量（SE）方程，随后基于比能量的变化规律，滚刀磨损机制和滚刀布局方式提出了新的TBM滚刀磨损预测模型，并在秦岭隧道的数据之上进行了验证16。yang 提出了一种基于磨粒磨损机制的滚刀磨损率和线性磨损率预测模型，该模型考虑了岩石的单轴抗压强度 （UCS） 和接触面条件，并且在广东狮子港隧道上得到的验证结果平均误差在10%以内17。除此之外，由于滚刀与岩石之间的磨损类型以磨料磨损为主，而DEM中的离散颗粒具有典型的磨粒磨损特性，能有效的模拟出滚刀与岩石的相互作用。因此，许多学者也使用DEM的方法来研究滚刀的磨损机制18-22。上述模型大多基于硬岩等特定地质开发，只适合特定地质条件下的滚刀磨损预测，但实际工程中，面对地质条件复杂或地质变化大的工况，难以实现准确预测，因此这种方法更多的是用来研究滚刀结构设计以及刀盘上滚刀的合理布置。

广泛的研究表明，TBM的掘进参数反映了岩石-机器相互作用的结果，并包含实时和相对准确的岩石信息，它们可以帮助间接获得滚刀磨损信息23-27。例如，Liu等使用改进的支撑向量机（SVR）算法建立了基于转速（RPM）等TBM掘进参数的岩体参数预测模型，验证了用掘进参数预测岩体参数的可行性25。除此之外，Jing等采用逐步回归法建立了使用单个滚刀的贯入速度和推力预测岩石单轴抗压强度（UCS）的模型，实现了对岩体状态的实时感知27。上述研究实现了用掘进参数预测岩石参数，间接的验证了用掘进参数预测滚刀磨损的可行性。而随着近年来机器学习技术的发展，由数据驱动的基于掘进参数的滚刀磨损预测模型受到了广泛关注28,29。例如，Mahmoodzadeh等使用高斯过程回归（GPR）、决策树（DT）、支持向量回归（SVR）和K-近邻（KNN）算法四种机器学习方法来预测TBM滚刀磨损，并针对特定案例验证了所建立模型的有效性30。同时，Elbaz等通过将GMDH（Group Method of Data Handling）神经网络和遗传算法集成建立了一个新的滚刀磨损预测模型，在滚刀磨损预测任务上的表现对比传统经验模型有显著提升29。机器学习方法在滚刀磨损预测任务中表现优异，然而，传统的机器学习技术往往只在传感器收集到的上百种数据中使用如力矩，转速等几个关键参数作为输入，忽略了其他参数如泡沫流量等也会对滚刀磨损产生影响，其对掘进数据利用率过低，同时过少的参数导致其构建的模型对复杂映射关系的表达能力不足，预测磨损量的准确率也有限。

相较于传统机器学习算法，深度学习可以在没有进行特征提取的情况下主动学习高维数据中的复杂关系，特别对于TBM，其记录的掘进参数有100余种，对于掘进参数和滚刀磨损之间这种高维数据间的复杂映射关系，十分适用深度学习方法建立隐式模型。例如，Agrawal等使用神经网络研究推力、扭矩、滚刀径向位置等掘进参数和单轴抗压强度（UCS），Cerchar磨损指数（CAI）等地质参数对滚刀磨损（CW）和钻进率（RoP）的影响规律，发现滚刀磨损与受CAI，UCS和推力影响更大，而RoP更多的取决于推力和扭矩24。Kilic等采用一维卷积神经网络来绘制滚刀的磨损量与现场掘进参数之间的关系，所建立模型在预测准确度和处理时间上都优于KNN，随机森林等6种传统机器学习方法以及LSTM，MLP网络32。Bai等通过对传统机器学习方法，MLP和时间序列模型等方法预测滚刀磨损准确度进行比较，得出在滚刀磨损预测任务中，深度学习方法相比机器学习方法更有效的结论，所建立的PCA + Min Max + GRU相较于其他模型能提供更好的预测结果33。上述几种模型使用了神经网络来建立滚刀磨损预测模型，并验证了在滚刀磨损预测任务中深度学习方法相比传统机器学习方法更有效。然而由于上述滚刀磨损预测模型都是基于监督学习方法构建的，这种方法的有效性很大程度上取决于标签的准确性。最直观的标签自然是滚刀磨损量，但是滚刀磨损量只能通过停机检查获得，在实际工程中一般只能获得几百条磨损量标签，样本量过少导致无法训练出准确的模型。

为解决标签量不足的问题，从而构建足够样本的数据集，学者们提出了两种解决思路。第一种方法是通过使用测量得到的磨损数据生成新的磨损值。例如，Ding等使用滚刀切口的螺旋轨迹长度估计了每个环的滚刀磨损，并将其定义为滚刀的磨损标签34。Wang等将掘进段设为1m，使用三次样条插值方法拟合得到每个掘进段的磨损量作为该掘进段的磨损标签5。Kwon等为解决训练集中异常磨损和正常磨损样本量的不平衡，使用合成少数过采样技术（SMOTA）生成新的数据集训练模型35。但是这种方法获得数据存在局限，一是用来生成数据的磨损数据本就因为人工测量有噪声，在此基础上进行插值得到的数据并不可靠。二是生成的样本无法准确代表真实世界的数据实例，从而可能引入对模型性能产生负面影响的噪声。第二种方法更为常用，即通过构建一个易获取的健康指标作为标签。 例如，Liu等将掘进过程看作“换刀-掘进-换刀”的过程，从而将预测滚刀磨损问题转换为预测滚刀更换的二分类问题，将停机换刀前一段时间的数据定义标签“0”，将掘进过程中的数据定义为标签“1”，所提出的模型在预测滚刀更换任务上可以获得90%的平均准确率34。Yu等将滚刀的掘进距离和最大掘进距离的比值定义为健康指数，并对刀盘上的每个滚刀进行归一化处理，从而表征了每个盘型滚刀的磨损，所建立模型在滚刀磨损预测任务上的平均准确率高达87.8%6。Elbaz等使用滚刀的挖掘体积与最大挖掘体积来构建滚刀磨损标签，所建立模型预测准确度高于经验模型29。这种方法的有效性则取决于健康指标设计的准确性。

1	Zhao, J.-L., Zhu, X.-Y., Jiang, C., Yang, S. & Xiao, X. Rock Adaptability of TBM Variable Cross-section Cutter Ring Based on Finite Element Simulation. Rock Mechanics and Rock Engineering 56, 3621–3639 (2023). https://doi.org/10.1007/s00603-023-03245-y

2	Yang, W., Chen, Z., Wu, G. & Xing, H. Probabilistic model of disc-cutter wear in TBM construction: A case study of Chaoer to Xiliao water conveyance tunnel in China. Science China Technological Sciences 66, 3534–3548 (2023). https://doi.org/10.1007/s11431-023-2465-y

3	Qin, C. et al. Precise cutterhead torque prediction for shield tunneling machines using a novel hybrid deep neural network. Mechanical Systems and Signal Processing 151, 107386 (2021). https://doi.org/https://doi.org/10.1016/j.ymssp.2020.107386

4	Hassanpour, J., Rostami, J., Tarigh Azali, S. & Zhao, J. Introduction of an empirical TBM cutter wear prediction model for pyroclastic and mafic igneous rocks; a case history of Karaj water conveyance tunnel, Iran. Tunnelling and Underground Space Technology 43, 222–231 (2014). https://doi.org/https://doi.org/10.1016/j.tust.2014.05.007

5	Wang, H. et al. A real-time multi-head mixed attention mechanism-based prediction method for tunnel boring machine disc cutter wear. Science China Technological Sciences 68, 1120302 (2024). https://doi.org/10.1007/s11431-024-2794-6

6	Yu, H. et al. A field parameters-based method for real-time wear estimation of disc cutter on TBM cutterhead. Automation in Construction 124, 103603 (2021). https://doi.org/https://doi.org/10.1016/j.autcon.2021.103603

7	Tao, J., Yu, H., Qin, C., Sun, H. & Liu, C. A gene expression programming-based method for real-time wear estimation of disc cutter on TBM cutterhead. Neural Computing and Applications 34, 20231–20247 (2022). https://doi.org/10.1007/s00521-022-07597-4

8	Gong, Q., Wu, F., Wang, D., Qiu, H. & Yin, L. Development and Application of Cutterhead Working Status Monitoring System for Shield TBM Tunnelling. Rock Mechanics and Rock Engineering 54, 1731–1753 (2021). https://doi.org/10.1007/s00603-021-02380-8

9	Trapp, M. & Nestorović, T. Non-gradient full waveform inversion approaches for exploration during mechanized tunneling applied to surrogate laboratory measurements. Tunnelling and Underground Space Technology 120, 104252 (2022). https://doi.org/https://doi.org/10.1016/j.tust.2021.104252

10	Gu, C., Liu, S., Chen, H., Bao, W. & Jiang, H. Design of Disc Cutter Wear Test System and Research on Wear Law Based on Eddy Current Testing Technology. Rock Mechanics and Rock Engineering 58, 2071–2088 (2025). https://doi.org/10.1007/s00603-024-04259-w

11	Lan, H., Xia, Y., Ji, Z., Fu, J. & Miao, B. Online monitoring device of disc cutter wear – Design and field test. Tunnelling and Underground Space Technology 89, 284–294 (2019). https://doi.org/https://doi.org/10.1016/j.tust.2019.04.010

12	Zhang, K. et al. Wear anomaly detection method of tunnel boring machine disc cutters based on anomaly-attention improved long short-term memory autoencoder. Measurement 253, 117497 (2025). https://doi.org/https://doi.org/10.1016/j.measurement.2025.117497

13	Hassanpour, J. Development of an empirical model to estimate disc cutter wear for sedimentary and low to medium grade metamorphic rocks. Tunnelling and Underground Space Technology 75, 90–99 (2018). https://doi.org/https://doi.org/10.1016/j.tust.2018.02.009

14	Barzegari, G., Khodayari, J. & Rostami, J. Evaluation of TBM Cutter Wear in Naghadeh Water Conveyance Tunnel and Developing a New Prediction Model. Rock Mechanics and Rock Engineering 54, 6281–6297 (2021). https://doi.org/10.1007/s00603-021-02640-7

15	Wang, L., Kang, Y., Zhao, X. & Zhang, Q. Disc cutter wear prediction for a hard rock TBM cutterhead based on energy analysis. Tunnelling and Underground Space Technology 50, 324–333 (2015). https://doi.org/https://doi.org/10.1016/j.tust.2015.08.003

16	Wang, L. et al. The energy method to predict disc cutter wear extent for hard rock TBMs. Tunnelling and Underground Space Technology 28, 183–191 (2012). https://doi.org/https://doi.org/10.1016/j.tust.2011.11.001

17	Yandong, Y., Kui, C., Fengyuan, L. & Jianjun, Z. Wear prediction model of disc cutter. Journal of China Coal Society 40, 1290–1296 (2015).

18	Zhang, X., Jiang, H., Tian, M., Li, H. & Chen, H. Rock cutting performance and wear characteristics of undercutting disc cutter: Based on experiment and DEM. Wear 528-529, 204976 (2023). https://doi.org/https://doi.org/10.1016/j.wear.2023.204976

19	Tang, M. et al. Study of the cutter-rock interaction mechanism during TBM tunnelling in mudstone: insight from DEM simulations of rotatory cutting tests. Bulletin of Engineering Geology and the Environment 81, 298 (2022). https://doi.org/10.1007/s10064-022-02785-x

20	Agrawal, A. K., Chattopadhyaya, S. & Murthy, V. M. S. R. Delineation of cutter force and cutter wear in different edge configurations of disc cutters – An analysis using discrete element method. Engineering Failure Analysis 129, 105727 (2021). https://doi.org/https://doi.org/10.1016/j.engfailanal.2021.105727

21	Labra, C., Rojek, J. & Oñate, E. Discrete/Finite Element Modelling of Rock Cutting with a TBM Disc Cutter. Rock Mechanics and Rock Engineering 50, 621–638 (2017). https://doi.org/10.1007/s00603-016-1133-7

22	Yang, H., Wang, H. & Zhou, X. Analysis on the Rock–Cutter Interaction Mechanism During the TBM Tunneling Process. Rock Mechanics and Rock Engineering 49, 1073–1090 (2016). https://doi.org/10.1007/s00603-015-0796-9

23	Fang, Y., Li, X., Guo, Y., Jin, D. & Liu, H. Effects of Driving Parameters on TBM Dynamic Response and Cracking of the Disc Cutter Ring: A Case Study. Rock Mechanics and Rock Engineering 57, 3063–3082 (2024). https://doi.org/10.1007/s00603-023-03716-2

24	Agrawal, A. K., Murthy, V. M. S. R., Chattopadhyaya, S. & Raina, A. K. Prediction of TBM Disc Cutter Wear and Penetration Rate in Tunneling Through Hard and Abrasive Rock Using Multi-layer Shallow Neural Network and Response Surface Methods. Rock Mechanics and Rock Engineering 55, 3489–3506 (2022). https://doi.org/10.1007/s00603-022-02834-7

25	Liu, B. et al. Improved support vector regression models for predicting rock mass parameters using tunnel boring machine driving data. Tunnelling and Underground Space Technology 91, 102958 (2019). https://doi.org/https://doi.org/10.1016/j.tust.2019.04.014

26	Yagiz, S. Utilizing rock mass properties for predicting TBM performance in hard rock condition. Tunnelling and Underground Space Technology 23, 326–339 (2008). https://doi.org/https://doi.org/10.1016/j.tust.2007.04.011

27	Jing, L.-j. et al. A case study of TBM performance prediction using field tunnelling tests in limestone strata. Tunnelling and Underground Space Technology 83, 364–372 (2019). https://doi.org/https://doi.org/10.1016/j.tust.2018.10.001

28	Wang, G., Singh, J., Tan, J. & Li, G. Use of predictive model for identification of overall wear state of TBM cutterhead based on tunneling parameters. Expert Systems with Applications 268, 126316 (2025). https://doi.org/https://doi.org/10.1016/j.eswa.2024.126316

29	Elbaz, K., Shen, S.-L., Zhou, A., Yin, Z.-Y. & Lyu, H.-M. Prediction of Disc Cutter Life During Shield Tunneling with AI via the Incorporation of a Genetic Algorithm into a GMDH-Type Neural Network. Engineering 7, 238–251 (2021). https://doi.org/https://doi.org/10.1016/j.eng.2020.02.016

30	Mahmoodzadeh, A. et al. Machine learning forecasting models of disc cutters life of tunnel boring machine. Automation in Construction 128, 103779 (2021). https://doi.org/https://doi.org/10.1016/j.autcon.2021.103779

31	Zhou, C., Gao, Y., Chen, E. J., Ding, L. & Qin, W. Deep learning technologies for shield tunneling: Challenges and opportunities. Automation in Construction 154, 104982 (2023). https://doi.org/https://doi.org/10.1016/j.autcon.2023.104982

32	Kilic, K., Toriya, H., Kosugi, Y., Adachi, T. & Kawamura, Y. One-Dimensional Convolutional Neural Network for Pipe Jacking EPB TBM Cutter Wear Prediction. Applied Sciences 12, 2410 (2022).

33	Bai, L., Mo, D., Li, H., Huang, W. & Cai, Z. Optimized Data Preprocessing and Model Selection for TBM Cutter Wear Prediction. Coatings 15, 564 (2025).

34	Liu, Y., Huang, S., Wang, D., Zhu, G. & Zhang, D. Prediction Model of Tunnel Boring Machine Disc Cutter Replacement Using Kernel Support Vector Machine. Applied Sciences 12, 2267 (2022).

35	Kwon, K., Choi, H., Jung, J., Kim, D. & Shin, Y. J. Prediction of abnormal TBM disc cutter wear in mixed ground condition using interpretable machine learning with data augmentation. Journal of Rock Mechanics and Geotechnical Engineering 17, 2059–2071 (2025). https://doi.org/https://doi.org/10.1016/j.jrmge.2024.05.027

公式实现

走通流程

# 磨损体积计算公式

## 1. 原始公式 (1-30号刀具)

### 正向：半径 → 体积

$$V = 2\pi \Delta R \cdot N \left[ TR - \frac{T}{2}\Delta R + R\tan(\theta)\Delta R - \frac{2}{3}\tan(\theta)(\Delta R)^2 \right]$$

其中：

- $V$: 磨损体积 (mm³)

- $\Delta R$: 磨损半径 (mm)

- $T$: 刀具厚度 (mm), 默认 25.4

- $R$: 刀具位置半径 (mm), 默认 241.3

- $\theta$: 刀具角度 (度), 默认 20°

- $N$: 刀具数量, 默认 1

### 反向：体积 → 半径

求解三次方程：

$$-\frac{2}{3}\tan(\theta)(\Delta R)^3 + \left[R\tan(\theta) - \frac{T}{2}\right](\Delta R)^2 + TR \cdot \Delta R - \frac{V}{2\pi N} = 0$$

**解法**：选择最小的正实根作为物理解

---

## 2. 新公式 (31-44号刀具)

### 参数定义

- $T = 25.4$ mm

- $H = 2413$ mm (刀具高度)

- $\theta = 20°$

- $d = (7-30) \times \frac{70}{13}$ (度)

### 中间变量计算

#### 计算 $h_0$:

$$h_0 = \frac{\left[\frac{T}{2} + \frac{R}{\tan\theta}\right] \cdot \tan(2\theta - d)}{1 - \tan(d)}$$

#### 计算 $h_1$:

$$h_1 = \left[\frac{R}{\tan\theta} - \frac{T}{2}\right] \cdot \left[1 - \frac{\sin\theta \cdot \cos\theta}{\sin(2\theta - d)}\right] + \tan\theta$$

### 判定条件

当 $h = 2R$ 时，计算判定值：

$$\text{condition} = \frac{h_0}{\tan\theta} - \frac{h_1}{\tan(2\theta - d)}$$

**判定规则**：

- 若 $\text{condition} > T_{pd}$ (其中 $T_{pd} = T$)，使用**公式1**

- 否则，使用**公式2**

### 公式1：$V = V_0 - V_1$

$$V_0 = \pi \left[\frac{R}{\tan\theta} + \frac{T}{2}\right] \cdot h_0 \cdot \left[H - \frac{h_0}{3}\right]$$

$$V_1 = \pi \left[\frac{R}{\tan\theta} - \frac{T}{2}\right] \cdot h_1 \cdot \left[H - \frac{h_1}{3}\right]$$

$$V = V_0 - V_1$$

### 公式2：原始公式（不含位置半径R参数）

$$V = 2\pi \Delta R \left[ T\Delta R - \frac{T}{2}\Delta R + \Delta R \tan(\theta) \Delta R - \frac{2}{3}\tan(\theta)(\Delta R)^2 \right]$$

简化为：

$$V = 2\pi \Delta R \left[ \frac{T}{2}\Delta R + \tan(\theta)(\Delta R)^2 - \frac{2}{3}\tan(\theta)(\Delta R)^2 \right]$$

$$V = 2\pi \Delta R \left[ \frac{T}{2}\Delta R + \frac{1}{3}\tan(\theta)(\Delta R)^2 \right]$$

### 反向：体积 → 半径

使用数值方法求解：

$$f(\Delta R) = V_{\text{calculated}}(\Delta R) - V_{\text{target}} = 0$$

**求解方法**：

1. 使用 `scipy.optimize.fsolve` 进行迭代求解

2. 如果失败，使用 `scipy.optimize.brentq` 在区间 $[0, 100]$ 内搜索

3. 初始猜测值：$\Delta R_0 = \sqrt{\frac{V}{2\pi T H}}$

---

## 3. 特殊规则

### 44号刀具

44号刀具的磨损量与43号刀具完全一致：

$$V_{44} = V_{43}$$

---

## 4. 公式对比

对于相同的磨损半径 $\Delta R$，两个公式计算的体积差异：

| $\Delta R$ (mm) | 原始公式 $V$ (mm³) | 新公式 $V$ (mm³) | 差异 (%) |

|-----------------|-------------------|-----------------|----------|

| 0.1             | 3,856             | 967,606         | 24,995%  |

| 0.5             | 19,373            | 1,066,547       | 5,405%   |

| 1.0             | 38,980            | 1,209,349       | 3,002%   |

| 2.0             | 78,895            | 1,558,686       | 1,876%   |

| 5.0             | 204,159           | 3,116,174       | 1,426%   |

| 10.0            | 430,776           | 7,407,718       | 1,620%   |

**结论**：新公式计算的体积远大于原始公式，这是因为新公式考虑了更大的刀具高度 $H = 2413$ mm。

---

## 5. 实现细节

### 数值稳定性处理

1. **避免除零**：

- 当 $\sin(2\theta - d) \approx 0$ 时，设置为 $10^{-10}$

2. **三次方程求解**：

- 选择最小的正实根（物理解）

- 实数判定：$|\text{Im}(root)| < 10^{-10}$

3. **单调性保证**：

- 使用累积最大值确保磨损量单调递增

- $V_{\text{corrected}}[i] = \max(V_{\text{corrected}}[i], V_{\text{corrected}}[i-1])$

4. **非负性约束**：

- $V = \max(0, V)$

- $\Delta R = \max(0, \Delta R)$